SOC Homelab Project

Portfolio Documentation

Version 1.0  |  March 2026

# Project Status

Network Segmentation (VLANs): ✅ Complete

Firewall (VyOS): ✅ Complete

SIEM (Wazuh): ✅ Complete

DNS Filtering (Pi-hole): ✅ Complete

Endpoint Monitoring: ⏳ Ongoing

Threat Detection Rules: ⏳ Ongoing

# Executive Summary

This project documents the design, implementation, and configuration of a fully functional Security Operations Center (SOC) homelab environment. The lab simulates a real enterprise network with multiple isolated network segments, a stateful firewall, centralized SIEM, and DNS-level threat filtering.

The environment was built from scratch using consumer hardware running Proxmox as a hypervisor, with multiple virtual machines and containers replicating production SOC infrastructure. The project demonstrates practical skills in network security, Linux administration, virtualization, and security monitoring.

# Skills Demonstrated

| Skill Area | Technology Used | Level |
| --- | --- | --- |
| Network Segmentation | VLANs, Managed Switch | Intermediate |
| Firewall Configuration | VyOS | Intermediate |
| SIEM Deployment | Wazuh | Intermediate |
| DNS Security | Pi-hole | Intermediate |
| Virtualization | Proxmox VE | Intermediate |
| Linux Administration | Ubuntu Server | Intermediate |
| Windows Administration | Windows 10 | Intermediate |
| Network Troubleshooting | TCP/IP, DHCP, NAT | Intermediate |
| Security Architecture | Zero-trust VLAN design | Intermediate |

# Network Architecture

## Physical Infrastructure

| Component | Device | Role |
| --- | --- | --- |
| Workstation | HP Laptop | Proxmox Hypervisor Host |
| Router | Tenda F3 | WAN / Internet Gateway |
| Switch | 8-Port Web Smart Switch | VLAN-aware switching |
| Workstation | Kali Linux Laptop | Security Testing (VLAN 20) |

## VLAN Design

| VLAN ID | Name | Subnet | Purpose | Security Level |
| --- | --- | --- | --- | --- |
| 1 | Management | 192.168.2.0/24 | Router & Switch Management | High |
| 10 | Servers | 10.10.10.0/24 | Wazuh, Ubuntu, Pi-hole | High |
| 20 | Security | 10.10.20.0/24 | Kali Linux Pentesting | Medium |
| 30 | Desktop | 10.10.30.0/24 | Windows 10 Workstation | Medium |

## Virtual Machines & Containers

| VM/CT | OS | IP Address | VLAN | Role |
| --- | --- | --- | --- | --- |
| Wazuh Server | Ubuntu 24.04 LTS | 10.10.10.101 | 10 | SIEM Manager |
| Ubuntu Server | Ubuntu 24.04 LTS | 10.10.10.100 | 10 | Wazuh Agent / Services |
| Pi-hole | Debian (LXC) | 10.10.10.2 | 10 | DNS Filtering |
| VyOS Firewall | VyOS 2026.02 | 10.10.10.1 | Trunk | Firewall / Router |
| Windows 10 | Windows 10 | 10.10.30.100 | 30 | Endpoint Agent |
| Kali Linux | Kali Linux | 10.10.20.x | 20 | Pentesting |

# Firewall Policy

## Inter-VLAN Traffic Rules

| Rule | Source | Destination | Action | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Any | Any | Accept | Established/Related traffic |
| 5 | VLAN 10 | VLAN 10 | Accept | Server-to-server communication |
| 10 | VLAN 10 | Internet | Accept | Server internet access |
| 20 | VLAN 20 | Any | Accept | Kali full access (pentest) |
| 25 | VLAN 30 | Wazuh (10.10.10.101) | Accept | Agent to SIEM communication |
| 26 | VLAN 30 | VLAN 10 | Drop | Block desktop from servers |
| 30 | VLAN 30 | Internet | Accept | Desktop internet access |
| 31 | VLAN 30 | Wazuh port 1514 | Accept | Wazuh agent traffic |
| 32 | VLAN 30 | Wazuh port 1515 | Accept | Wazuh enrollment |
| Default | Any | Any | Drop | Deny all other traffic |

# SIEM Configuration (Wazuh)

## Monitored Endpoints

| Agent ID | Hostname | IP Address | OS | Status |
| --- | --- | --- | --- | --- |
| 000 | wazuh-manager | 127.0.0.1 | Ubuntu 24.04 | Active/Local |
| 004 | ubuntu-server | 10.10.10.100 | Ubuntu 24.04 | Active |
| 003 | DESKTOP-46UHFOD | 10.10.30.100 | Windows 10 | Active |

## Wazuh Capabilities Deployed

- Log collection from Windows and Linux endpoints

- Real-time security event monitoring

- File integrity monitoring (FIM)

- Vulnerability detection

- Active response capabilities

# DNS Security (Pi-hole)

Pi-hole is deployed as the DNS server for all VLANs, providing network-wide ad blocking and DNS-level threat filtering. All DNS queries from monitored endpoints are logged and visible in the Pi-hole dashboard.

## Pi-hole Configuration

- IP Address: 10.10.10.2 (static)

- Upstream DNS: 8.8.8.8, 1.1.1.1

- Listening on: All interfaces (serves all VLANs)

- DNS queries from VLAN 10, 20, 30 all route through Pi-hole

# Challenges & Solutions

| Challenge | Solution | Lesson Learned |
| --- | --- | --- |
| pfSense installer loop (Netgate) | Switched to OPNsense, then VyOS | Open-source alternatives often work better |
| OPNsense disk partition error | Switched to VyOS (Linux-based) | BSD-based OS can have Proxmox disk issues |
| IPFire NIC detection failure | Changed NIC model to Intel E1000 | VirtIO drivers not always available in installer |
| VLAN 1 switch modification rejected | Used PVID settings instead | Some switches lock VLAN 1 permanently |
| VyOS DHCP DNS syntax error | Used resolvectl and resolv.conf instead | Always check VyOS version-specific syntax |
| Windows agent wrong IP after VLAN | Deleted old agent, re-registered | Agent config must match new network layout |
| Inter-VLAN firewall rule order | Moved specific allow rules before drop rule | Firewall rules are processed top-down |

# Key Learnings

## Technical Skills Gained

- Designed and implemented enterprise-grade VLAN segmentation from scratch

- Configured stateful firewall with granular inter-VLAN rules using VyOS

- Deployed and configured Wazuh SIEM with multi-platform agent monitoring

- Troubleshot complex network issues across physical and virtual infrastructure

- Learned practical differences between pfSense, OPNsense, IPFire, and VyOS

- Configured Pi-hole as a network-wide DNS security layer

## Security Concepts Applied

- Network segmentation and zero-trust architecture

- Defense in depth — firewall + SIEM + DNS filtering

- Principle of least privilege in firewall rules

- Centralized logging and monitoring

- Endpoint detection and response (EDR) fundamentals

# Tools & Technologies

| Category | Tool/Technology | Version | Purpose |
| --- | --- | --- | --- |
| Hypervisor | Proxmox VE | 8.x | Virtual machine host |
| Firewall | VyOS | 2026.02 | Network firewall & routing |
| SIEM | Wazuh | 4.14.4 | Security monitoring |
| DNS | Pi-hole | Latest | DNS filtering |
| OS (Server) | Ubuntu Server | 24.04 LTS | Server workloads |
| OS (Security) | Kali Linux | 2024.x | Penetration testing |
| OS (Desktop) | Windows 10 | Latest | Endpoint monitoring |
| Switching | Web Smart Switch | 8-port | VLAN segmentation |

# Planned Improvements

## Short Term

- Connect Kali Linux laptop to switch Port 3 for VLAN 20 isolation

- Install Wazuh agent on Kali Linux

- Install Wazuh agent on Pi-hole container

- Configure VyOS logs forwarding to Wazuh

- Fix Windows 10 DHCP (currently static IP)

## Medium Term

- Deploy Suricata IDS on VyOS for network threat detection

- Create custom Wazuh detection rules for homelab

- Set up Wazuh dashboards for SOC monitoring practice

- Configure pfSense/OPNsense when second NIC arrives

- Add IoT VLAN (VLAN 40) for future smart devices

## Long Term

- Simulate real attack scenarios using Kali Linux

- Practice incident response using Wazuh alerts

- Add Elastic Stack for advanced log analysis

- Deploy honeypot on isolated VLAN

- Document attack simulations for portfolio

# Resume / CV Entry

Copy and paste this into your CV/resume under Projects:

SOC Homelab Lab Environment  |  2025–2026

Designed and built a fully segmented SOC homelab environment on Proxmox VE:

- Implemented multi-VLAN network segmentation using a managed switch and VyOS firewall, isolating servers, security tools, and endpoints into separate network zones

- Deployed Wazuh SIEM with agents monitoring Windows 10 and Ubuntu Server endpoints, enabling centralized log collection and real-time security event monitoring

- Configured stateful firewall rules enforcing zero-trust principles between VLANs, with Pi-hole providing DNS-level threat filtering across all network segments

- Troubleshot and resolved complex networking issues including firewall rule ordering, DHCP across VLANs, and cross-platform Wazuh agent registration

Technologies: Proxmox VE | VyOS | Wazuh | Pi-hole | Ubuntu Server | Kali Linux | Windows 10 | VLANs | Firewall | SIEM