SOC Homelab Project

Portfolio Documentation — Version 3.0

July 2026  |  Jimmy Mwendwa  |  Nairobi, Kenya

# What's New in Version 3.0

- VLAN 40 Honeypot — Cisco EPC3928S WiFi lure + OpenCanary VM

- Dedicated Proxmox management NIC (USB ethernet → vmbr1)

- Ubuntu 24.04 LTS on laptop (replaced Kali Linux)

- 6 Wazuh agents all active including OpenCanary honeypot

- Custom detection rules 100001 and 100002 written and tested

- Full attack simulation lab — 5 attack scenarios executed

- 5 incident reports written in professional SOC format

- File Integrity Monitoring enabled on /etc/

- False positive investigation and analysis completed

- Wazuh detection rules study notes PDF created

# Project Status

Network Segmentation (VLANs 10/20/30/40): ✅ Complete

Firewall (VyOS): ✅ Complete

SIEM (Wazuh): ✅ Complete

DNS Filtering (Pi-hole): ✅ Complete

Honeypot (Cisco + OpenCanary): ✅ Complete

All 6 Wazuh Agents Active: ✅ Complete

Custom Detection Rules: ✅ Complete

Attack Simulation Lab: ✅ Complete

Incident Reports (5): ✅ Complete

GitHub Portfolio: ✅ Updated v3

LinkedIn Profile: ✅ Published

VyOS Log Forwarding: ⏳ In Progress

Active Directory Lab: ⏳ Planned

# Network Architecture

| VLAN | Name | Subnet | Gateway | Purpose |
| --- | --- | --- | --- | --- |
| 10 | Servers | 10.10.10.0/24 | 10.10.10.1 | Wazuh, Ubuntu Server, Pi-hole |
| 20 | Security | 10.10.20.0/24 | 10.10.20.1 | Ubuntu laptop — attack/defend |
| 30 | Desktop | 10.10.30.0/24 | 10.10.30.1 | Windows 10 endpoint |
| 40 | Honeypot | 192.168.40.0/24 | 192.168.40.1 | Cisco FreeWiFi + OpenCanary VM |

# VM & Container Inventory

| VM/CT | OS | Static IP | VLAN | Role | Agent |
| --- | --- | --- | --- | --- | --- |
| Wazuh Server | Ubuntu 24.04 | 10.10.10.101 | 10 | SIEM Manager | Local ✅ |
| Ubuntu Server | Ubuntu 24.04 | 10.10.10.100 | 10 | Endpoint | Active ✅ |
| Pi-hole | Debian LXC | 10.10.10.2 | 10 | DNS Filtering | Active ✅ |
| VyOS Firewall | VyOS 2026.02 | 10.10.10.1 | Trunk | Firewall/Router | N/A |
| Windows 10 | Windows 10 | 10.10.30.100 | 30 | Endpoint | Active ✅ |
| Ubuntu Laptop | Ubuntu 24.04 | 10.10.20.x | 20 | Attack/Defend | Active ✅ |
| OpenCanary VM | Ubuntu 24.04 | 192.168.40.x | 40 | Honeypot VM | Active ✅ |
| Cisco EPC3928S | Firmware | 192.168.40.2 | 40 | WiFi Lure AP | N/A |

# Honeypot Architecture — VLAN 40

## Layer 1 — Cisco EPC3928S WiFi Lure

- Broadcasts SSID: "FreeWiFi" — open network, no password

- Devices connect and get isolated on VLAN 40 (192.168.40.0/24)

- VyOS DHCP assigns 192.168.40.100+ to connected devices

- Completely isolated from real servers — firewall rules 56/57 only allow Wazuh ports

## Layer 2 — OpenCanary VM Fake Services

| Service | Port | Simulates | MITRE |
| --- | --- | --- | --- |
| SSH | 2222 | Linux SSH server | T1021.004 |
| FTP | 21 | File transfer server | T1071.002 |
| HTTP | 80 | Web login page | T1071.001 |
| MySQL | 3306 | Database server | T1505.001 |
| RDP | 3389 | Windows Remote Desktop | T1021.001 |
| Telnet | 23 | Legacy remote access | T1021 |

# Wazuh SIEM — 6 Agents Active

| Agent ID | Hostname | IP | OS | Status | Added |
| --- | --- | --- | --- | --- | --- |
| 000 | wazuh-manager | 127.0.0.1 | Ubuntu 24.04 | Active/Local ✅ | Phase 1 |
| 003 | DESKTOP-46UHFOD | 10.10.30.100 | Windows 10 | Active ✅ | Phase 1 |
| 004 | ubuntu-server | 10.10.10.100 | Ubuntu 24.04 | Active ✅ | Phase 1 |
| 005 | pihole | 10.10.10.2 | Debian | Active ✅ | Phase 2 |
| 006 | ubuntu-laptop | 10.10.20.x | Ubuntu 24.04 | Active ✅ | Phase 3 |
| 007 | opencanary | 192.168.40.x | Ubuntu 24.04 | Active ✅ | Phase 3 |

# Custom Detection Rules

| Rule ID | Description | Level | MITRE |
| --- | --- | --- | --- |
| 100001 | SSH brute force — 5 failures in 60s from same IP | 10 High | T1110 |
| 100002 | Successful SSH login after brute force — possible compromise | 15 Critical | T1110, T1078 |

# Attack Simulation Lab — Incident Reports

| IR # | Attack Type | Rule Fired | Level | Outcome | MITRE |
| --- | --- | --- | --- | --- | --- |
| IR-001 | SSH Brute Force + Successful Login | 100001, 100002 | 10, 15 | CRITICAL — Compromise confirmed | T1110, T1078 |
| IR-002 | CVE-2025-45582 Vulnerability Detected | 23504 | 7 | HIGH — Package needs update | T1068 |
| IR-003 | Network Reconnaissance (Nmap scan) | 5402 | 3 | MEDIUM — Context elevated severity | T1046 |
| IR-004 | File Integrity Alert (/etc/ tampered) | 510 | 7 | HIGH — FIM alert fired | T1543 |
| IR-005 | Trojan detection /usr/bin/diff | 510 | 7 | FALSE POSITIVE — Confirmed clean | T1574 |

## Key SOC Lessons Learned

- Alert level alone does not determine severity — context and correlation matter more

- Alert correlation: same IP doing brute force then nmap = high priority regardless of individual rule levels

- if_matched_sid requires frequency/timeframe to work correctly in chained rules

- Always search existing Wazuh rules before writing custom ones — avoid duplicates

- False positive investigation is a core SOC skill — dpkg --verify confirmed /usr/bin/diff was legitimate

- FIM must be configured as realtime="yes" to catch active attacks, not just scheduled scans

# Infrastructure Upgrade — Dedicated Management NIC

| Before Upgrade | After Upgrade |
| --- | --- |
| nic0 → vmbr0 carries BOTH Proxmox mgmt AND VyOS traffic | nic0 → vmbr0 carries VyOS traffic ONLY |
| Single point of failure for entire homelab | USB NIC → vmbr1 → Proxmox management (192.168.2.150) |
| VyOS crash = Proxmox inaccessible | VyOS crash = Proxmox still accessible via vmbr1 |
| Shared bandwidth between management and VMs | Dedicated bandwidth per function |

# Challenges & Solutions (v3 New)

| Challenge | Root Cause | Solution | Lesson |
| --- | --- | --- | --- |
| Rule 100002 not triggering | if_matched_sid without frequency = no counting | Added frequency="2" timeframe="120" | Always pair if_matched_sid with frequency for chained detections |
| Honeypot reaching real servers | Rule 53 allowed VLAN 40 → VLAN 10 | Deleted rule 53, added rules 56/57 (Wazuh ports only) | Honeypot isolation must be verified after setup |
| FIM not realtime | Default scan every 12 hours (43200s) | Added realtime="yes" to /etc/ syscheck config | FIM must be realtime to catch active attacks |
| False positive on /usr/bin/diff | Wazuh rootcheck had outdated hash entry | dpkg --verify + hash comparison confirmed clean | Validate every alert — not all alerts are real threats |
| GitHub auth failed | Password auth no longer supported | Created Personal Access Token (ghp_...) | Always use tokens not passwords for Git operations |

# Resume / CV Entry — Version 3

Copy and paste into your CV under Projects:

SOC Homelab Lab Environment  |  2025–2026

- Designed and deployed enterprise-grade SOC homelab on Proxmox VE with multi-VLAN segmentation, VyOS stateful firewall, Wazuh SIEM monitoring 6 endpoints, Pi-hole DNS filtering, and two-layer honeypot (Cisco WiFi lure + OpenCanary VM)

- Simulated 5 real attack scenarios (SSH brute force, CVE exploitation, network reconnaissance, file tampering) and investigated each alert using professional incident response methodology with MITRE ATT&CK mapping

- Wrote custom Wazuh detection rules (100001-100002) for SSH brute force and post-compromise detection at level 15 critical severity; independently debugged and fixed rule logic when initial version failed to trigger

- Produced 5 formal incident reports with attack timeline, impact assessment, MITRE techniques, response actions and lessons learned — demonstrating professional SOC analyst workflow

Technologies: Proxmox VE · VyOS · Wazuh · Pi-hole · OpenCanary · Ubuntu 24.04 · Windows 10 · VLANs · FIM · MITRE ATT&CK · Hydra · Nmap · Cisco EPC3928S

# Portfolio Links

| Platform | Link |
| --- | --- |
| GitHub | https://github.com/munchydevante-bot/SOC-Homelab |
| LinkedIn | https://www.linkedin.com/in/jimmy-mwendwa-581985379 |
| Location | Nairobi, Kenya |
| Open to Work | SOC Analyst, Junior Security Analyst, IT Security roles |