SOC Homelab Project

Portfolio Documentation — Version 2.0

April 2026  |  Jimmy Mwendwa  |  Nairobi, Kenya

# What's New in Version 2.0

- Kali Linux Wazuh agent installed and active

- Pi-hole agent installed and active

- All 5 endpoints now monitored by Wazuh SIEM

- Static IPs configured on all VMs (no more DHCP conflicts)

- VyOS intra-VLAN firewall rules fixed for inter-VM communication

- Wazuh IOC files permanent fix via cron job

- VM auto-start configured in Proxmox

- LinkedIn profile published with SOC portfolio

- GitHub repository published with full documentation

# Project Status

Network Segmentation (VLANs): ✅ Complete

Firewall (VyOS): ✅ Complete

SIEM (Wazuh): ✅ Complete

DNS Filtering (Pi-hole): ✅ Complete

Endpoint Monitoring (All 5 agents): ✅ Complete

Static IP Configuration: ✅ Complete

Remote Access (Tailscale): ✅ Complete

Wazuh Detection Rules: ⏳ In Progress

Attack Simulation: ⏳ In Progress

VyOS Log Forwarding: ⏳ In Progress

# Network Architecture

## VLAN Design

| VLAN ID | Name | Subnet | Gateway | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Management | 192.168.2.0/24 | 192.168.2.1 | Router & Switch Mgmt |
| 10 | Servers | 10.10.10.0/24 | 10.10.10.1 | Wazuh, Ubuntu, Pi-hole |
| 20 | Security | 10.10.20.0/24 | 10.10.20.1 | Kali Linux Pentesting |
| 30 | Desktop | 10.10.30.0/24 | 10.10.30.1 | Windows 10 Workstation |

## VM & Container Inventory

| VM/CT | OS | Static IP | VLAN | Role | Wazuh Agent |
| --- | --- | --- | --- | --- | --- |
| Wazuh Server | Ubuntu 24.04 | 10.10.10.101 | 10 | SIEM Manager | Local |
| Ubuntu Server | Ubuntu 24.04 | 10.10.10.100 | 10 | Agent/Services | Active ✅ |
| Pi-hole | Debian LXC | 10.10.10.2 | 10 | DNS Filtering | Active ✅ |
| VyOS Firewall | VyOS 2026.02 | 10.10.10.1 | Trunk | Firewall/Router | N/A |
| Windows 10 | Windows 10 | 10.10.30.100 | 30 | Endpoint | Active ✅ |
| Kali Linux | Kali Linux | 10.10.20.x | 20 | Pentesting | Active ✅ |

# Wazuh SIEM — All Agents Active

| Agent ID | Hostname | IP | OS | Status | Added |
| --- | --- | --- | --- | --- | --- |
| 000 | wazuh-manager | 127.0.0.1 | Ubuntu 24.04 | Active/Local ✅ | Initial |
| 003 | DESKTOP-46UHFOD | 10.10.30.100 | Windows 10 | Active ✅ | Phase 1 |
| 004 | ubuntu-server | 10.10.10.100 | Ubuntu 24.04 | Active ✅ | Phase 1 |
| 005 | pihole | 10.10.10.2 | Debian | Active ✅ | Phase 2 |
| 006 | kali-linux | 10.10.20.x | Kali Linux | Active ✅ | Phase 2 |

# Wazuh Detection Rules (MITRE ATT&CK Mapped)

| Rule ID | Name | Level | MITRE Technique | Platform |
| --- | --- | --- | --- | --- |
| 100001 | SSH Brute Force (5 attempts/60s) | 10 | T1110 - Brute Force | Linux |
| 100002 | Login Success After Brute Force | 12 | T1110 - Brute Force | Linux |
| 100003 | New Linux User Created | 8 | T1136 - Create Account | Linux |
| 100004 | Sudo Privilege Escalation | 6 | T1548 - Abuse Elevation | Linux |
| 100005 | Windows Brute Force | 8 | T1110 - Brute Force | Windows |
| 100006 | New Windows User Created | 10 | T1136 - Create Account | Windows |
| 100007 | Nmap Port Scan Detected | 10 | T1046 - Network Discovery | All |
| 100008 | /etc/ File Modified | 8 | T1543 - Modify System Process | Linux |
| 100009 | Direct Root Login | 12 | T1078 - Valid Accounts | Linux |
| 100010 | Same IP Brute Force (10/120s) | 10 | T1110 - Brute Force | Linux |

# VyOS Firewall Policy

| Rule | Source | Destination | Action | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Any | Any | Accept | Established/Related traffic |
| 5 | VLAN 10 | VLAN 10 | Accept | Intra-server communication |
| 10 | VLAN 10 | Internet | Accept | Server internet access |
| 20 | VLAN 20 | Any | Accept | Kali full access |
| 25 | VLAN 30 | Wazuh .101 | Accept | Agent to SIEM |
| 26 | VLAN 30 | VLAN 10 | Drop | Block desktop from servers |
| 30 | VLAN 30 | Internet | Accept | Desktop internet access |
| 31 | VLAN 30 | Wazuh:1514 | Accept | Wazuh agent traffic |
| 32 | VLAN 30 | Wazuh:1515 | Accept | Wazuh enrollment |
| 40 | 192.168.2.0/24 | VLAN 10 | Accept | WiFi to VLAN 10 |
| 41 | VLAN 10 | 192.168.2.0/24 | Accept | VLAN 10 return traffic |
| 42 | 192.168.2.0/24 | Pi-hole:53 | Accept | WiFi DNS to Pi-hole |
| Default | Any | Any | Drop | Deny all other traffic |

# Pi-hole DNS Security

| Setting | Value |
| --- | --- |
| IP Address | 10.10.10.2 (static) |
| Upstream DNS | 8.8.8.8, 1.1.1.1 |
| Listening Mode | All interfaces |
| Domains Blocked | 264,935+ |
| Blocklists | StevenBlack, Disconnect.me, Hagezi Pro |
| Wazuh Agent | Active ✅ |

# Challenges & Solutions (Updated)

| Challenge | Root Cause | Solution |
| --- | --- | --- |
| IP conflict (Kali & VyOS both .101) | DHCP assigned same IP | Changed VyOS WAN to static .102 |
| Wazuh manager crashing on reboot | Missing IOC list files | Cron job creates files on @reboot |
| Ubuntu can't reach Wazuh after reboot | Two IPs on same interface (.100 & .101) | Deleted 50-cloud-init.yaml DHCP config |
| Pi-hole not accepting external queries | listeningMode set to LOCAL | Changed to ALL in pihole.toml |
| VyOS firewall blocking intra-VLAN traffic | No rule for VLAN 10 → VLAN 10 | Added rule 5 accepting 10.10.10.0/24 |
| Wazuh agents disconnecting after reboot | VMs not getting correct static IPs | Set static IPs via netplan on all VMs |
| Tailscale subnet routes not working | IP forwarding disabled on Proxmox | Enabled net.ipv4.ip_forward=1 |

# Stability Improvements

## Auto-start Configuration

- VyOS Firewall → Start at boot: YES, Order: 1

- Pi-hole CT → Start at boot: YES, Order: 2

- Wazuh Server → Start at boot: YES, Order: 3

- Ubuntu Server → Start at boot: YES, Order: 4

- Windows 10 → Start at boot: YES, Order: 5

## Static IP Configuration

- Wazuh Server: 10.10.10.101/24 (netplan static)

- Ubuntu Server: 10.10.10.100/24 (netplan static)

- Pi-hole: 10.10.10.2/24 (container static)

- VyOS WAN: 192.168.2.100/24 (static)

- Windows 10: 10.10.30.100/24 (manual static)

## Wazuh IOC Permanent Fix

- Cron job (@reboot) creates missing IOC files before Wazuh starts

- Prevents Wazuh manager crash on every reboot

- Files: malware-hashes, malicious-ip, malicious-domains

# Planned Next Steps

## Short Term

- Add Wazuh detection rules (in progress)

- Forward VyOS firewall logs to Wazuh

- Simulate attacks from Kali Linux

- Practice SOC analyst workflow

## Medium Term

- Deploy Suricata IDS on VyOS

- Create custom Wazuh dashboards

- Set up TheHive for incident ticketing

- Deploy Atomic Red Team for attack simulation

## Long Term

- Complete CompTIA Security+ certification

- Complete TryHackMe SOC Level 1 path

- Document attack simulations for portfolio

- Apply for SOC Analyst positions

# Portfolio & Contact

| Platform | Link |
| --- | --- |
| GitHub | https://github.com/munchydevante-bot/SOC-Homelab |
| LinkedIn | https://www.linkedin.com/in/jimmy-mwendwa-581985379 |
| Location | Nairobi, Kenya |
| Open to Work | SOC Analyst, Security Analyst, IT Security roles |